APB TO I2C BRIDGE IC

Features

- Supports up to 32-bit wide data width

- Half- duplex transmission

- Operating frequencies – 100KHz, 400KHz, 1MHz, 3.3MHz

- 7-bit slave address, up to 127 slaves

- Repeated start available

Pin description

Input pin’s

- PRESETn- Active High, synchronous RESET

- PCLK-Peripheral Clock (100 MHz)

- PSEL- Peripheral select for APB_I2C_IC

- PENABLE- Data Transmission ENABLE

- PADDR-Configuration Bits (32-bit wide)

- PWrite-Enable Read/ Write Operation (1 -Write, 0-Read)

- PWDATA-Data Input (32-bit wide)

Output pin’s

- PREADY-APB Slave Output to indicate that APB-Slave is ready for next transaction (to SoC)

- PRDATA-APB Slave data Output (32-bit wide) (to SoC)

- PSLVERR- APB Slave Output to indicate error in transaction (to SoC)

- I2c_scl – Serial Clock Line for I2C devices, bidirectional, pull-up required (to I2C bus)

- I2c_sda- Serial Data Line for I2C devices bidirectional, pull-up required (to I2C bus)

Note: All pins are 1-bit wide, unless mentioned.

Description

The I2C is a protocol which helps to communicate the external devices (RTC, temperature, voltage regulator etc.), sensors, data storage devices (EPROMs).

The I2C protocol half- duplex, 2-wire simple protocol.

Register Description

I2C Control Registers:

I2C con1: This register controls the transaction parameters of the I2C master. It is 8-bit length [7:0].

I2C con1[0]- it sets or resets the master

I2C con1[1]- it enables the master controller for clock, for clock gating and power saving

I2C con1[2:3]- byte-count, indicates transaction length-8,16,24,32 bit

I2C con1[4]- it indicates that the data send was a data, or an address location

I2C con1[5]- it is set a transaction with repeated start (for page reading in EEPROMs)

I2C con1[6:7]- it sets the frequency of the Serial clock line, 2’b00-100KHz, 2’b01- 400KHz, 2’b10-                1MHz, 2’b11-3.3MHz.

I2C con2: This register has the slave address and the read/write bit. It is 8 bit length [7:0]

I2C con2[0]- read/write bit

I2C con2[1:6]- 7-bit slave address.

I2C Status Register:

I2C stat: This register indicates the status of the I2C master. It is 8-bit length [7:0].

I2C con1[0]- it indicates if the master is busy in a transaction

I2C con1[1]- it indicates whether the master is in a read cycle or write cycle.

I2C con1[2:3]- byte-count, indicates the transaction length-8,16,24,32 bit.

I2C con1[4]- it indicates that whether the master is in start state.

I2C con1[5]- it indicates whether the master is in data sending or receiving state.

I2C con1[6]- it indicates whether the master has received a the acknowledge or not.

I2C con1[7]- it indicates that whether the master is in stop state

Protocol operation

The pin configuration as follows to perform read/write operations:

- To check status of I2C-master:

APB-read to read status of I2C master: Useful for knowing the status of I2C-master, because I2C is very slow protocol compared to APB, hence knowing its status before issuing a transaction is essential, because it prevents over-writing. I2C status registers indicate whether the I2C master is in transaction, along with the information of transaction parameter, where is it in the transaction timeline.

PRESETn = 1;

PSEL = 1;

PENABLE = 1;

PWrite = 0;

PADDR = 32'h0000_0000; // ZERO in all 32-bit indicates the data is sent/received to/from I2C-control and status registers.

PRDATA <= 0000_0000_{I2C-STAT}_{I2C-CON2}_{I2C-CON2};

- To write data to I2C-slave device:

APB-write to configure I2C master: To configure the I2C control registers, the parameters are set using PWDATA, I2C-control are selected using PADDR.

PRESETn = 1;

PSEL = 1;

PENABLE = 1;

PWrite = 1;

PADDR = 32'h0000_0000; // ZERO in all 32-bit indicates the data is sent/received to/from I2C-control and status registers.

//con2                     con1

|addr 0-6| |r/w|  |ff   R  D/A  cc   e   r|

PWDATA = 32'b0000_0000    1100_011    0        11_0__1__00_1_1; // PWDATA[0:7] -> CON1

// PWDATA[8:15] -> CON2

APB-write to send data to I2C master: To send data using PWDATA, Din is selected using PADDR.

PRESETn = 1;

PSEL = 1;

PENABLE = 1;

PWrite = 1;

PADDR = 32'hffff_0000; // NON-ZERO value indicates the data is sent/received to/from Din and Dout registers.

PWDATA = 32'habcd_ef12; // PWDATA[0:31] -> Din[0:31]

- To read data form I2C-Slave devices:

APB-write to configure I2C master: To configure the I2C control registers, the parameters are set using PWDATA, I2C-control are selected using PADDR.

PRESETn = 1;

PSEL = 1;

PENABLE = 1;

PWrite = 1;

PADDR = 32'h0000_0000; // ZERO in all 32-bit indicates the data is sent/received to/from I2C-control and status registers.

//con2                     con1

|addr 0-6| |r/w|  |ff   R  D/A  cc   e   r|

PWDATA = 32'b0000_0000    1100_011    1        11_0__1__00_1_1; // PWDATA[0:7] -> CON1

// PWDATA[8:15] -> CON2

APB-read to read status of read transaction: for knowing the status of read, because I2C is very slow protocol compared to APB, hence knowing its status before reading the data from APB-slave is essential. If I2C status registers indicate that the reading form I2C-Slave is completed then a data read cycle is issued (APB data read).

PRESETn = 1;

PSEL = 1;

PENABLE = 1;

PWrite = 0;

PADDR = 32'h0000_0000; // ZERO in all 32-bit indicates the data is sent/received to/from I2C-control and status registers.

PRDATA <= 0000_0000_{I2C-STAT}_{I2C-CON2}_{I2C-CON2};

Note: Time delay to issue the APB-read for data read is predicted by the status of I2C-master in transaction time-line, frequency of operation, transaction length.

APB-read to read data from I2C master: To read data using PRDATA, Dout is selected using PADDR. Done after a time delay.

PRESETn = 1;

PSEL = 1;

PENABLE = 1;

PWrite = 1;

PADDR = 32'hffff_0000; // NON-ZERO value indicates the data is sent/received to/from Din and Dout registers.

PRDATA[0:31] <= Dout[0:31] ;

Tested board:

I2C protocol tested using artix-7 board with following chip device.

Pin Description

For FPGA (Arty-7) implementation on board toggle switches are used to simulate signals from APB-bus. Single color LEDs for address, PREADY. RGB LEDs for 8-bit DATA register. Buttons for indexing of 32-bit DATA.

When, PWrite = 1, RGB LEDs -> PWDATA [Index] [0:7], BUTTONs -> Index

When, PWrite = 0, RGB LEDs -> PRDATA [Index] [0:7], BUTTONs -> Index

| NET | Type | Port Number |
| --- | --- | --- |
| PCLK | Input | E3 (CLK 100MHz, internal pin) |
| PRESETn | Input | A10 (SW3) |
| PSEL | Input | C10 (SW2) |
| PENABLE | Input | C11 (SW1) |
| PWrite | Input | A8 (SW0) |
| BUTTON [0] | Input | D9 (BTN0) |
| BUTTON [1] | Input | C9 (BTN1) |
| BUTTON [2] | Input | B9 (BTN2) |
| BUTTON [3] | Input | B8 (BTN3) |
| ADDR [0] | Output | J5 (LED5) |
| ADDR [1] | Output | T9 (LED6) |
| ADDR [2] | Output | T10 (LED7) |
| PREADY | Output | H5 (LED4) |
| DATA [7] | Output | F6 (led0_g) |
| DATA [6] | Output | J4 (led1_g) |
| DATA [5] | Output | J2 (led2_g) |
| DATA [4] | Output | H6 (led3_g) |
| DATA [3] | Output | E1 (led0_b) |
| DATA [2] | Output | G4 (led1_b) |
| DATA [1] | Output | H4 (led2_b) |
| DATA [0] | Output | K2 (led3_b) |
| I2C-SCL | Inout (bidirectional) | L18 (scl) |
| I2C-SDA | Inout (bidirectional) | M18 (sda) |

Circuit Connections:

Bus connection:

Output:

Reading from I2C slave device:

Writing to I2C slave device: